| P/N: | 81.1BD42.BE20B (48.26219.0020B) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F/W: | ADMAC21A |  |  |  |  |  |  |
| Flash Type: | Micron MT29F16G08CBACAWP:C |  |  |  |  |  |  |
| Description: | AFD187-M 8GB |  |  |  |  |  |  |
| Controller | 65.00289.133FB |  |  |  |  |  |  |
| Test Result: | Pass |  |  |  |  |  |  |
| Bugs and limitations: |  |  |  |  |  |  |  |
| Prepared By: | Martin Chiang |  |  |  |  |  |  |
| Approval By: | Danny Chen |  |  |  |  |  |  |
| Date: | 2012/09/14 |  |  |  |  |  |  |
| File Name: | 81.1BD42.BE20B AFD187-M 8GB (ADMAC21A) |  |  |  |  |  |  |
| System Configuration |  |  |  |  |  |  |  |
| M/B/ Chipset | ASUS M3A78-EM / AMD780G + SB700 |  |  |  |  |  |  |
| AFD 187-M | 8GB |  |  |  |  |  |  |
| Test Program result |  |  |  |  |  |  |  |
| Function test (Windows platform) |  | Win XP |  |  |  |  |  |
|  |  | PASS |  |  |  | FAIL |  |
| Data access |  |  |  |  |  |  |  |
| Data Delete |  |  |  |  |  |  |  |
| Data Compare |  |  |  |  |  |  |  |
| Restart |  |  |  |  |  |  |  |
| Suspend |  |  |  |  |  |  |  |
| Format Size |  | FAT32: 7.43GB |  |  |  | NTFS: 7.45GB |  |
| Install Windos CE System |  |  |  | Pass |  |  |  |
| Install Windos XP-E |  |  |  | Pass |  |  |  |
| Install Windos 2000 SP4 System |  |  |  | Pass |  |  |  |
| Install Windos XP SP2 System |  |  |  | Pass |  |  |  |
| Install Linux (2.6.29.4-167.fc11)System |  |  |  | Pass |  |  |  |
| Rebooter Test 1000 Cycle |  |  |  | Pass |  |  |  |
| Long-run Iometer(2006.07.27-RC3)-15hr |  |  |  | Pass |  |  |  |
| Long-run BurnIn Test V4.0 Pro-48hr |  |  |  | Pass |  |  |  |
| Comp32 – 15hr |  |  |  | Pass |  |  |  |
| Speed test Unit =KB/s |  |  |  |  |  |  |  |
| HD Bench 34b6 | Read |  | Write |  | Random read |  | Random write |
| Win XP (FAT32) | 97709 |  | 23151 |  | 72778 |  | 6488 |
| Win XP (NTFS) | 97709 |  | 20868 |  | 71210 |  | 6507 |

| System Configuration |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M/B/ Chipset | Gigabyte GA-8I955X Pro / Intel 955X + ICH7R |  |  |  |  |  |  |
| AFD 187-M | 8GB |  |  |  |  |  |  |
| Test Program result |  |  |  |  |  |  |  |
| Function test (Windows platform) |  | Win XP |  |  |  |  |  |
|  |  | PASS |  |  |  | FAIL |  |
| Data access |  |  |  |  |  |  |  |
| Data Delete |  |  |  |  |  |  |  |
| Data Compare |  |  |  |  |  |  |  |
| Restart |  |  |  |  |  |  |  |
| Suspend |  |  |  |  |  |  |  |
| Format Size |  | FAT32: 7.43GB |  |  |  | NTFS: 7.45GB |  |
| Install Windos CE System |  |  |  | Pass |  |  |  |
| Install Windos XP-E |  |  |  | Pass |  |  |  |
| Install Windos 2000 SP4 System |  |  |  | Pass |  |  |  |
| Install Windos XP SP2 System |  |  |  | Pass |  |  |  |
| Install Linux (2.6.29.4-167.fc11)System |  |  |  | Pass |  |  |  |
| Rebooter Test 1000 Cycle |  |  |  | Pass |  |  |  |
| Long-run Iometer(2006.07.27-RC3)-15hr |  |  |  | Pass |  |  |  |
| Long-run BurnIn Test V4.0 Pro-48hr |  |  |  | Pass |  |  |  |
| Comp32 – 15hr |  |  |  | Pass |  |  |  |
| Speed test Unit =KB/s |  |  |  |  |  |  |  |
| HD Bench 34b6 | Read |  | Write |  | Random read |  | Random write |
| Win XP (FAT32) | 82847 |  | 23073 |  | 61798 |  | 6159 |
| Win XP (NTFS) | 85049 |  | 20932 |  | 63602 |  | 6203 |

| System Configuration |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M/B/ Chipset | Asus P5GD1-VM / Intel 915G + ICH6 |  |  |  |  |  |  |
| AFD 187-M | 8GB |  |  |  |  |  |  |
| Test Program result |  |  |  |  |  |  |  |
| Function test (Windows platform) |  | Win XP |  |  |  |  |  |
|  |  | PASS |  |  |  | FAIL |  |
| Data access |  |  |  |  |  |  |  |
| Data Delete |  |  |  |  |  |  |  |
| Data Compare |  |  |  |  |  |  |  |
| Restart |  |  |  |  |  |  |  |
| Suspend |  |  |  |  |  |  |  |
| Format Size |  | FAT32: 7.43GB |  |  |  | NTFS: 7.45GB |  |
| Install Windos CE System |  |  |  | Pass |  |  |  |
| Install Windos XP-E |  |  |  | Pass |  |  |  |
| Install Windos 2000 SP4 System |  |  |  | Pass |  |  |  |
| Install Windos XP SP2 System |  |  |  | Pass |  |  |  |
| Install Linux (2.6.29.4-167.fc11)System |  |  |  | Pass |  |  |  |
| Rebooter Test 1000 Cycle |  |  |  | Pass |  |  |  |
| Long-run Iometer(2006.07.27-RC3)-15hr |  |  |  | Pass |  |  |  |
| Long-run BurnIn Test V4.0 Pro-48hr |  |  |  | Pass |  |  |  |
| Comp32 – 15hr |  |  |  | Pass |  |  |  |
| Speed test Unit =KB/s |  |  |  |  |  |  |  |
| HD Bench 34b6 | Read |  | Write |  | Random read |  | Random write |
| Win XP (FAT32) | 99128 |  | 21481 |  | 71960 |  | 6130 |
| Win XP (NTFS) | 100688 |  | 19734 |  | 72778 |  | 5979 |

- Additional Qualification:

- Environmental

- RoHS Standard _

- Other Environmental Standard

- WEEE (By Product)

- Battery Standard

- Packing Standard

- Others

- Certification

- CE     No.

- FCC    No.

- BSMI   No.

- VCCC   No.

- C-tick   No.

- Other