| P/N: | 81.1BD82.BE20B (48.26219.0020B) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F/W: | ADMAC21A |  |  |  |  |  |  |
| Flash Type: | Micron MT29F16G08CBACAWP:C |  |  |  |  |  |  |
| Description: | AFD187-M 16GB |  |  |  |  |  |  |
| Controller | 65.00289.133FB |  |  |  |  |  |  |
| Test Result: | Pass |  |  |  |  |  |  |
| Bugs and limitations: |  |  |  |  |  |  |  |
| Prepared By: | Martin Chiang |  |  |  |  |  |  |
| Approval By: | Danny Chen |  |  |  |  |  |  |
| Date: | 2012/09/14 |  |  |  |  |  |  |
| File Name: | 81.1BD82.BE20B AFD187-M 16GB (ADMAC21A) |  |  |  |  |  |  |
| System Configuration |  |  |  |  |  |  |  |
| M/B/ Chipset | ASUS M2V-TVM / Via K8M890 + VT8237R Plus |  |  |  |  |  |  |
| AFD 187-M | 32GB |  |  |  |  |  |  |
| Test Program result |  |  |  |  |  |  |  |
| Function test (Windows platform) |  | Win XP |  |  |  |  |  |
|  |  | PASS |  |  |  | FAIL |  |
| Data access |  |  |  |  |  |  |  |
| Data Delete |  |  |  |  |  |  |  |
| Data Compare |  |  |  |  |  |  |  |
| Restart |  |  |  |  |  |  |  |
| Suspend |  |  |  |  |  |  |  |
| Format Size |  | FAT32: 14.8GB |  |  |  | NTFS: 14.9GB |  |
| Install Windos CE System |  |  |  | Pass |  |  |  |
| Install Windos XP-E |  |  |  | Pass |  |  |  |
| Install Windos 2000 SP4 System |  |  |  | Pass |  |  |  |
| Install Windos XP SP2 System |  |  |  | Pass |  |  |  |
| Install Windos Vista System |  |  |  | Pass |  |  |  |
| Install Windos 7 System |  |  |  | Pass |  |  |  |
| Install Linux (2.6.29.4-167.fc11)System |  |  |  | Pass |  |  |  |
| Install WES 7 System |  |  |  | Pass |  |  |  |
| Rebooter Test 1000 Cycle |  |  |  | Pass |  |  |  |
| Long-run Iometer(2006.07.27-RC3)-15hr |  |  |  | Pass |  |  |  |
| Long-run BurnIn Test V4.0 Pro-48hr |  |  |  | Pass |  |  |  |
| Comp32 – 15hr |  |  |  | Pass |  |  |  |
| Speed test Unit =KB/s |  |  |  |  |  |  |  |
| HD Bench 34b6 | Read |  | Write |  | Random read |  | Random write |
| Win XP (FAT32) | 109168 |  | 44560 |  | 80820 |  | 8891 |
| Win XP (NTFS) | 112775 |  | 42261 |  | 79875 |  | 8679 |

| System Configuration |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M/B/ Chipset | GIGABYTE EP45-UD3LR / Intel P45 + ICH10R |  |  |  |  |  |  |
| AFD 187-M | 16GB |  |  |  |  |  |  |
| Test Program result |  |  |  |  |  |  |  |
| Function test (Windows platform) |  | Win XP |  |  |  |  |  |
|  |  | PASS |  |  |  | FAIL |  |
| Data access |  |  |  |  |  |  |  |
| Data Delete |  |  |  |  |  |  |  |
| Data Compare |  |  |  |  |  |  |  |
| Restart |  |  |  |  |  |  |  |
| Suspend |  |  |  |  |  |  |  |
| Format Size |  | FAT32: 14.8GB |  |  |  | NTFS: 14.9GB |  |
| Install Windos CE System |  |  |  | Pass |  |  |  |
| Install Windos XP-E |  |  |  | Pass |  |  |  |
| Install Windos 2000 SP4 System |  |  |  | Pass |  |  |  |
| Install Windos XP SP2 System |  |  |  | Pass |  |  |  |
| Install Windos Vista System |  |  |  | Pass |  |  |  |
| Install Windos 7 System |  |  |  | Pass |  |  |  |
| Install Linux (2.6.29.4-167.fc11)System |  |  |  | Pass |  |  |  |
| Install WES 7 System |  |  |  | Pass |  |  |  |
| Rebooter Test 1000 Cycle |  |  |  | Pass |  |  |  |
| Long-run Iometer(2006.07.27-RC3)-15hr |  |  |  | Pass |  |  |  |
| Long-run BurnIn Test V4.0 Pro-48hr |  |  |  | Pass |  |  |  |
| Comp32 – 15hr |  |  |  | Pass |  |  |  |
| Speed test Unit =KB/s |  |  |  |  |  |  |  |
| HD Bench 34b6 | Read |  | Write |  | Random read |  | Random write |
| Win XP (FAT32) | 109052 |  | 44872 |  | 76190 |  | 8939 |
| Win XP (NTFS) | 110942 |  | 42827 |  | 76190 |  | 8831 |

| System Configuration |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M/B/ Chipset | MSI MS-7388(K9A2 CF) / AMD790X + SB600 |  |  |  |  |  |  |
| AFD 187-M | 16GB |  |  |  |  |  |  |
| Test Program result |  |  |  |  |  |  |  |
| Function test (Windows platform) |  | Win XP |  |  |  |  |  |
|  |  | PASS |  |  |  | FAIL |  |
| Data access |  |  |  |  |  |  |  |
| Data Delete |  |  |  |  |  |  |  |
| Data Compare |  |  |  |  |  |  |  |
| Restart |  |  |  |  |  |  |  |
| Suspend |  |  |  |  |  |  |  |
| Format Size |  | FAT32: 14.8GB |  |  |  | NTFS: 14.9GB |  |
| Install Windos CE System |  |  |  | Pass |  |  |  |
| Install Windos XP-E |  |  |  | Pass |  |  |  |
| Install Windos 2000 SP4 System |  |  |  | Pass |  |  |  |
| Install Windos XP SP2 System |  |  |  | Pass |  |  |  |
| Install Windos Vista System |  |  |  | Pass |  |  |  |
| Install Windos 7 System |  |  |  | Pass |  |  |  |
| Install Linux (2.6.29.4-167.fc11)System |  |  |  | Pass |  |  |  |
| Install WES 7 System |  |  |  | Pass |  |  |  |
| Rebooter Test 1000 Cycle |  |  |  | Pass |  |  |  |
| Long-run Iometer(2006.07.27-RC3)-15hr |  |  |  | Pass |  |  |  |
| Long-run BurnIn Test V4.0 Pro-48hr |  |  |  | Pass |  |  |  |
| Comp32 – 15hr |  |  |  | Pass |  |  |  |
| Speed test Unit =KB/s |  |  |  |  |  |  |  |
| HD Bench 34b6 | Read |  | Write |  | Random read |  | Random write |
| Win XP (FAT32) | 96240 |  | 45169 |  | 68956 |  | 8622 |
| Win XP (NTFS) | 99224 |  | 47440 |  | 70426 |  | 8764 |

- Additional Qualification:

- Environmental

- RoHS Standard _

- Other Environmental Standard

- WEEE (By Product)

- Battery Standard

- Packing Standard

- Others

- Certification

- CE     No.

- FCC    No.

- BSMI   No.

- VCCC   No.

- C-tick   No.

- Other