[image: image4.jpg]Apacer

Access the best





PAGE  



Product Qualification

--- Compatibility Test ---

	Product Name:
	M.2 PT42 32GB supported UEFI mode

	Product P/N:
	81.D2E31.ET100BA

	Firmware Version:
	SFPS925A

	Description:
	M.2 PT42 32GB supported UEFI mode

	Controller:
	ASMedia ASM1062 QFN64 C-grade

	Flash Type:
	uSDC-M Plus 32GB

	PCB Version:
	48.26287.0100B

	Qualification:
	Qual All

	Test Result:
	PASS


	APPROVAL

	QT
	Date
	Signature
	Approved by
	Date
	Signature

	Martin Chiang
	2017/02/17
	Martin Chiang
	Danny Chen
	2017/02/17
	Danny Chen


Table Of Content

3SUMMARY RESULT


4A.
Function Test With M/B : Gigabyte GA-Z170X Gaming 3


41.
Basic Function Test


42.
Format Test


43.
Burn In Test


44.
Reboot Test


45.
Install System Test


46.
Test Result


5B.
Function Test With M/B : Asus H97M-E


51.
Basic Function Test


52.
Reboot Test


53.
Install System Test


54.
Test Result


6C.
Performance Test With M/B : Gigabyte GA-Z170X Gaming 3


7D.
Additional Qualification:


71.
Environmental


72.
Certification




SUMMARY RESULT
	
	PC1
	PC2
	PC3
	PC4

	Testing
Product
	M.2 PT42 32GB supported UEFI mode
	
	

	System
	Windows 10 Enterprise
	Windows embedded standard 7
	
	

	M/B
	Name
	Z170-A
	H97M-E
	
	

	
	Vendor
	ASUS
	ASUS
	
	

	
	Chipset
	Intel Z170
	Intel H97
	
	

	
	Interface
	SATA III
	SATA III
	
	

	
	BIOS
	1702
	2602
	
	

	Test Result

	Basic Function Test
	V
	V
	
	

	Burn

In

Test
	Comp32
	V
	--
	
	

	
	PassMark

Burn In Test

Pro V8.0
	V
	--
	
	

	
	PassMark

Burn In Test

Pro V3.0.1007
	--
	--
	
	

	Warm Boot Test
	V
	--
	
	

	Cold Boot Test
	--
	V
	
	

	Install System Test

	
	Windows

Emb. 7
Standard
	Windows

Emb. 8

Standard
	Windows

Emb. 8.1
Industry
	Windows

10
	Fedora

kernel 3.3.4 -5 fc17
	Ubuntu

kernel
3.13.0 -32
	Red Hat

kernel
2.6.32 -431.e16

	Result
	V
	V
	V
	V
	V
	V
	V

	Remark:
Test Result — PASS



A. Function Test With M/B : Gigabyte GA-Z170X Gaming 3
	Testing Product
	M.2 PT42 32GB supported UEFI mode
	Testing Engineer
	Martin Chiang

	Testing Environment
	M/B:
	Gigabyte
	Chipset:
	Intel Z170
	BIOS:
	F5b

	
	
	GA-Z170X Gaming 3
	Interface:
	SATA III


1. Basic Function Test

	
	Copy
	Delete
	Restart
	Compare
	Suspend
(S3)
	Hibernate
(S4)
	LED

	Windows 10 Enterprise
	V
	V
	V
	V
	V
	V
	V


2. Format Test

	
	FAT32
	NTFS

	Windows 10 Enterprise
	29.7GB
	29.8GB


3. Burn In Test

	
	Comp32

15 hours
	Burn In Test V8.0 Pro
168 hours

	Windows 10 Enterprise
	V
	V


4. Reboot Test

	
	Warm Boot 6000 Cycles

	Windows 10 Enterprise
	V


5. Install System Test

	Install System
	Result

	Windows Embedded Standard 7
	V

	Windows Embedded 8 Standard
	V

	Windows Embedded 8.1 Industry Pro
	V

	Windows 10 Enterprise
	V

	Windows 10 Multiple Edition
	V


6. Test Result
	Pass




B. Function Test With M/B : Asus H97M-E
	Testing Product
	M.2 PT42 32GB supported UEFI mode
	Testing Engineer
	Martin Chiang

	Testing Environment
	M/B:
	Asus
	Chipset:
	Intel H97
	BIOS:
	2602

	
	
	H97M-E
	Interface:
	SATA III


1. Basic Function Test

	
	Copy
	Delete
	Restart
	Compare
	Suspend
(S3)
	Hibernate
(S4)
	LED

	Windows Embedded 8.1 Industry Pro
	V
	V
	V
	V
	V
	V
	V


2. Reboot Test

	
	Cold Boot 6000 Cycles

	Windows Embedded 8.1 Industry Pro
	V


3.  Install System Test

	Install System
	Result

	Windows Embedded Standard 7
	V

	Windows Embedded 8 Standard
	V

	Windows Embedded 8.1 Industry Pro
	V

	Windows 10 Enterprise
	V

	Windows 10 Multiple Edition
	V

	Fedora kernel 3.3.4 -5
	V

	Ubuntu kernel 3.13.0 -32
	V

	Red Hat Kernel 2.6.32 -431.e16
	V


4. Test Result
	Pass




C. Performance Test With M/B : Gigabyte GA-Z170X Gaming 3

	Testing Product
	M.2 PT42 32GB supported UEFI mode
	Testing Engineer
	Martin Chiang

	Testing Environment
	M/B:
	Gigabyte
	Chipset:
	Intel Z170
	BIOS:
	F5b

	
	
	GA-Z170X Gaming 3
	Interface:
	SATA III


	OS : Windows 10 / Performance Tool : CrystalDiskMark 5.1.2

	Before the Test
	After the Test

	[image: image1.png]= CrystalDiskMark 5.1.2 464 - X
Fie Setfings Theme Help Language

[5 v][16B_ w][E: 0% (0/30GiB) v
Read [MB/s]  Write [MB/s]

Wi 1333.0 1214.6
winl12183.9 18398
#4352/7 1106.7
#129.02 |57{51

All





	[image: image2.png]=5 CrystalDiskMark 5.1.2 64 - X

Eie Setting:

All

seq
qzssT!

4
Q256TI

Seq

4K

s Tneme Help Langusge

[5 v][16B_ w][E: 0% (0/30GiB) V]

Read [MB/s]  Write [MB/s]

383.4 \112.1
189.0 184.31
3813 1114.9
29.77 168{93







	OS : Ubuntu 15.01 / Performance Tool : Benchmark

	Performance Before the Test

	[image: image3.png]s0mws 1ms
s 03ms
a1 ws o8ms
384w o7ms
mwms osms
20mms o0sms
208 s 4 oams

03ms

o2ms

o1ms

oms
008

Disk or Device Partition 1 of 32 GB Disk — 32GB SATA Flash Drive [SFPS925A] (/dev/sdb1)
Last Benchmarked Fri 17 Feb 2017 01:53:10 PM CST (Less than a minute ago)
SampleSize 10.0 MiB (10,485,760 bytes)
Average Read Rate 485.7 MB/s (100 samples)
Average Write Rate 272.5MBs (100 samples)

Average Access Time 0.15 msec (1000 samples)

Start Benchmark. Close






D. Additional Qualification:

1. Environmental

1. RoHS Standard  2011/65/EU / PCRD005                              
2. Other Environmental Standard  REACH / China RoHS                   

3. WEEE (By Product)  2012/19/EU                                    
4. Battery Standard  N/A                                             
5. Packing Standard  94/62/EC                                        
6. Others  N/A                                                     
2. Certification

1. ( CE     No.                                                
2. ( FCC    No.                                                
3. ( BSMI   No.                                                
4. ( VCCC   No.                                               
5. ( C-tick   No.                                                
6. ( Other                                                      
1
 Form No: PCRD002-F05-02
PAGE  
2
 Form No: PCRD002-F05-02

