[image: image4.jpg]Apacer

Access the best




PAGE  


Product Qualification

--- Compatibility Test ---

	Product Name:
	SFD 18A-M 128GB 5V PCBA

	Product P/N:
	80.M1ET8.1T3B0BB (80.M1ET8.1T4B0BB)

	Firmware Version:
	SFDH103B_SVN536_141027

	Description:
	SFD 18A-M 128GB 5V PCBA

	Controller:
	JMF667H

	Flash Type:
	TH58TEG7DDKBA4C DDP

	PCB Version:
	48.28294.0100B

	Qualification:
	Qual FW

	Test Result:
	Pass


	APPROVAL

	QT
	Date
	Signature
	Approved by
	Date
	Signature

	Eric Cheng
	2015/01/21
	Eric Cheng
	Danny Chen
	2015/01/21
	Danny Chen


Table Of Content

3SUMMARY RESULT


4A.
Function Test With M/B : Asus M3A78-EM


41.
Basic Function Test


42.
Format Test


43.
Burn In Test


44.
Reboot Test


45.
Install System Test


46.
Test Result


5B.
Function Test With M/B : Asus J1900I-C


51.
Basic Function Test


52.
Format Test


53.
Burn In Test


54.
Reboot Test


55.
Install System Test


56.
Test Result


6C.
Function Test With M/B : Gigabyte EP45-UD3LR


61.
Basic Function Test


62.
Format Test


63.
Burn In Test


64.
Reboot Test


65.
Install System Test


66.
Test Result


7D.
Function Test With M/B : Asus AM1M-A


71.
Burn In Test


72.
Test Result


8E.
Function Test With M/B : Avalue EMX-QM77


81.
Reboot Test


82.
Power Failure Test


83.
Test Result


9F.
Performance Test With M/B : Asus X99-A


10G.
Performance Test With M/B : Asus X99-A


11H.
Platform Compatibility Test


12I.
Additional Qualification:


121.
Environmental


122.
Certification




SUMMARY RESULT
	
	PC1
	PC2
	PC3
	PC4
	PC5

	Testing
Product
	SFD 18A-M 128GB 5V PCBA

	System
	Windows 7
Linux
	Windows 7
Windows 8
	Windows 7
Windows 8
	Windows 7
	Linux

	M/B
	Name
	M3A78-EM
	J1900I-C
	EP45-UD3LR
	AM1M-A
	EMX-QM77

	
	Vendor
	Asus
	Asus
	Gigabyte
	Asus
	Avalue

	
	Chipset
	AMD 780G+SB700
	Intel Celeron J1900
	Intel P45+ICH10R
	AMD Sempron & Athlon-Seris APUs
	Intel QM77

	
	Interface
	SATA II
	SATA II
	SATA II
	SATA III
	SATA III

	
	BIOS
	2701
	0608
	F11
	0502
	4.6.5

	Test Result

	Basic Function Test
	V
	V
	V
	--
	--

	Burn

In

Test
	Comp32
	V
	V
	V
	--
	--

	
	Iometer

2006.07.27RC3
	V
	V
	V
	--
	--

	
	PassMark

Burn In Test

Pro V6.0.1018
	V
	V
	V
	--
	--

	Warm Boot Test
	V
	V
	V
	--
	--

	Cold Boot Test
	--
	--
	--
	--
	V

	Power Failure Test
	--
	--
	--
	--
	V

	Temp 60℃
Burn In Test
	--
	--
	--
	V
	--

	Install System Test

	
	Windows

CE
	Windows

XP-E
	Windows

Emb. 7
Standard
	Windows

Emb. 8.1
Industry
	Windows

XP
	Windows

7
	Windows

8.1
	Fedora

17
	Ubuntu

12.04
	Red Hat

6.3

	Result
	V
	V
	V
	V
	V
	V
	V
	V
	V
	V

	Remark:
Test Result — PASS



A. Function Test With M/B : Asus M3A78-EM
	Testing Product
	SFD 18A-M 128GB 5V PCBA
	Testing Engineer
	Eric Cheng

	Testing Environment
	M/B:
	Asus
	Chipset:
	AMD 780G+SB700
	BIOS:
	2701

	
	
	M3A78-EM
	Interface:
	SATA II


1. Basic Function Test

	
	Copy
	Delete
	Restart
	Compare
	Suspend
(S3)
	Hibernate
(S4)
	LED

	Win 7
	V
	V
	V
	V
	V
	V
	V

	Linux
	V
	V
	V
	V
	V
	V
	V


2. Format Test

	
	FAT32
	NTFS
	Ext4

	Win 7
	--
	119GB
	--

	Linux
	--
	--
	125.9GB


3. Burn In Test

	
	Comp32

15 hours
	Iometer(2006.07.27-RC3)
15 hours
	Burn In Test V6.0 Pro
48 hours

	Win 7
	V
	V
	V


4. Reboot Test

	
	Warm Boot 2000 Cycles

	Win 7
	V


5. Install System Test
	Install System
	Result

	Install Windows CE System
	V

	Install Windows XP-E System
	V

	Install Windows Embedded Standard 7 System
	V

	Install Windows Embedded 8 Standard System
	V

	Install Windows Embedded 8.1 Industry Pro System
	V

	Install Windows XP System
	V

	Install Windows 7 System
	V

	Install Windows 8.1 System
	V

	Install Fedora 17 System
	V

	Install Ubuntu 12.04 System
	V

	Install Red Hat 6.3 System
	V


6. Test Result
	Pass




B. Function Test With M/B : Asus J1900I-C
	Testing Product
	SFD 18A-M 128GB 5V PCBA
	Testing Engineer
	Eric Cheng

	Testing Environment
	M/B:
	Asus
	Chipset:
	Intel Celeron J1900
	BIOS:
	0608

	
	
	J1900I-C
	Interface:
	SATA II


1. Basic Function Test

	
	Copy
	Delete
	Restart
	Compare
	Suspend
(S3)
	Hibernate
(S4)
	LED

	Win 7
	V
	V
	V
	V
	V
	V
	V

	Win 8
	V
	V
	V
	V
	V
	V
	V


2. Format Test

	
	FAT32
	NTFS
	exFAT

	Win 7
	--
	119GB
	--

	Win 8
	--
	119GB
	--


3. Burn In Test

	
	Comp32

15 hours
	Iometer(2006.07.27-RC3)
15 hours
	Burn In Test V6.0 Pro
48 hours

	Win 7
	V
	V
	V


4. Reboot Test

	
	Warm Boot 2000 Cycles

	Win 7
	V


5. Install System Test
	Install System
	Result

	Install Windows CE System
	V

	Install Windows Embedded Standard 7 System
	V

	Install Windows Embedded 8 Standard System
	V

	Install Windows Embedded 8.1 Industry Pro System
	V

	Install Windows XP System
	V

	Install Windows 7 System
	V

	Install Windows 8.1 System
	V

	Install Fedora 17 System
	V

	Install Ubuntu 12.04 System
	V

	Install Red Hat 6.3 System
	V


6. Test Result
	Pass




C. Function Test With M/B : Gigabyte EP45-UD3LR
	Testing Product
	SFD 18A-M 128GB 5V PCBA
	Testing Engineer
	Eric Cheng

	Testing Environment
	M/B:
	Gigabyte
	Chipset:
	Intel P45+ICH10R
	BIOS:
	F11

	
	
	EP45-UD3LR
	Interface:
	SATA II


1. Basic Function Test

	
	Copy
	Delete
	Restart
	Compare
	Suspend
(S3)
	Hibernate
(S4)
	LED

	Win 7
	V
	V
	V
	V
	V
	V
	V

	Win 8
	V
	V
	V
	V
	V
	V
	V


2. Format Test

	
	FAT32
	NTFS
	exFAT

	Win 7
	--
	119GB
	--

	Win 8
	--
	119GB
	--


3. Burn In Test

	
	Comp32

15 hours
	Iometer(2006.07.27-RC3)
15 hours
	Burn In Test V6.0 Pro
48 hours

	Win 7
	V
	V
	V


4. Reboot Test

	
	Warm Boot 2000 Cycles

	Win 7
	V


5. Install System Test
	Install System
	Result

	Install Windows CE System
	V

	Install Windows XP-E System
	V

	Install Windows Embedded Standard 7 System
	V

	Install Windows Embedded 8 Standard System
	V

	Install Windows Embedded 8.1 Industry Pro System
	V

	Install Windows XP System
	V

	Install Windows 7 System
	V

	Install Windows 8.1 System
	V

	Install Fedora 17 System
	V

	Install Ubuntu 12.04 System
	V

	Install Red Hat 6.3 System
	V


6. Test Result
	Pass




D. Function Test With M/B : Asus AM1M-A

	Testing Product
	SFD 18A-M 128GB 5V PCBA
	Testing Engineer
	Eric Cheng

	Testing Environment
	M/B:
	Asus
	Chipset:
	AMD Sempron & Athlon-Seris APUs
	BIOS:
	0502

	
	
	AM1M-A
	Interface:
	SATA III


1. Burn In Test

	
	Burn In Test V6.0 Pro
Temp 60℃, 48 hours

	Win 7
	V


2. Test Result
	Pass




E. Function Test With M/B : Avalue EMX-QM77
	Testing Product
	SFD 18A-M 128GB 5V PCBA
	Testing Engineer
	Eric Cheng

	Testing Environment
	M/B:
	Avalue
	Chipset:
	Intel QM77
	BIOS:
	4.6.5

	
	
	EMX-QM77
	Interface:
	SATA III


1. Reboot Test

	
	Cold Boot 2000 Cycle

	Linux
	V


2. Power Failure Test

	
	2000 Cycles

	Linux
	V


3. Test Result
	Pass




F. Performance Test With M/B : Asus X99-A
	Testing Product
	SFD 18A-M 128GB 5V PCBA
	Testing Engineer
	Eric Cheng

	Testing Environment
	M/B:
	Asus
	Chipset:
	Intel X99
	BIOS:
	1004

	
	
	X99-A
	Interface:
	SATA III

	Testing Software
	CrystalMark 3.0.2



	Performance Before the Test
	Performance After the Test

	[image: image1.jpg]5 CrystalDiskMark 302 x64 (=] =

Ele Edit Theme Help Langusge

5 [+] [1000MB [~] [D: 0% (0/119GB) =]
Read [MB/s]  Write [MB/s]

= 512/1 3334
s 448.5 337.7
414133 196:54
W 135.2 1286.7

All





	[image: image2.jpg]CrystelDiskMark 3.0.2 64 o S

-
Ele Edit Theme Help Langusge
- 5 [+] [1000MB [~] [D: 0% (0/119GB) =]

Read [MB/s]  Write [MB/s]

= 524.6 326.8
s 447./9 330.3
% 141(27 195.05
&% 318.0 281.5







G. Performance Test With M/B : Asus X99-A
	Testing Product
	SFD 18A-M 128GB 5V PCBA
	Testing Engineer
	Eric Cheng

	Testing Environment
	M/B:
	Asus
	Chipset:
	Intel X99
	BIOS:
	1004

	
	
	X99-A
	Interface:
	SATA III

	Testing Software
	Ubuntu 12.01
Benchmark


	Performance Before the Test

	[image: image3.jpg]sk (ATA 128GB SATA Flash Drive) - Benchmark

0 mes ms
sioms 09ms
somes o8ms
aoms o7ms
0mes osms
0ms osms
20ms oams
160 o3ms
0w o02ms
oues o1ms
O W mw W% a% se e % % o k™

Minimum Read Rate: ~ 557.2Mb/s Minimum Write Rate: -

Maximum Read Rate: 564.3 Mb/s Maximum Write Rate: —

Average Read Rate:  560.2 Mb/s Average Write Rate: -

Last Benchmark Lessthanaminuteago Average Access Time: 0.0ms

S start Read-Only Benchmark & start Read/Write Benchmark
Meastre read rate and access time Measure read rate, wite rate and accesstime

| close







H. Platform Compatibility Test
	Platform Information
	Test Item

	Chipset
	M/B
	Interface
	System
	Format
	Copy
	Delete
	Compare
	Restart
	Suspend

	Intel P45
	Gigabyte EP45-UD3LR
	SATA II
	Win 7
	V
	V
	V
	V
	V
	V

	Intel P55
	Gigabyte P55A-UD5
	SATA III
	Win XP
	V
	V
	V
	V
	V
	V

	Intel Z68
	Asus P8Z68-M Pro
	SATA III
	Win 7
	V
	V
	V
	V
	V
	V

	Intel Z77
	Gigabyte GA-IVB
	SATA III
	Win 7
	V
	V
	V
	V
	V
	V

	Intel X99
	Asus X99-A
	SATA III
	Win 7
	V
	V
	V
	V
	V
	V

	Intel Celeron J1900
	Asus J1900I-C
	SATA II
	Win 7
	V
	V
	V
	V
	V
	V

	Intel NM70 Express
	Avalue EMX-NM70
	SATA III
	Win 7
	V
	V
	V
	V
	V
	V

	AMD Sempron & Athlon-Seris APUs
	Asus AM1M-A
	SATA III
	Win 7
	V
	V
	V
	V
	V
	V

	AMD A55E FCH
	Axiomtek MANO120
	SATA III
	Win 7
	V
	V
	V
	V
	V
	V

	AMD 780G
	Asus M3A78-EM
	SATA II
	Win 7
	V
	V
	V
	V
	V
	V


I. Additional Qualification:

1. Environmental

1. RoHS Standard                                                  
2. Other Environmental Standard                                      

3. WEEE (By Product)                                              
4. Battery Standard                                                 
5. Packing Standard                                                 
6. Others                                                         
2. Certification

1. ( CE     No.                                                
2. ( FCC    No.                                                
3. ( BSMI   No.                                                
4. ( VCCC   No.                                               
5. ( C-tick   No.                                                
6. ( Other                                                      
1
 Form No: PCRD002-F05-02
PAGE  
2
 Form No: PCRD002-F05-02

