# Product Qualification

# --- Compatibility Test ---

| Product Name: | SFD 25A-M 32GB (ET) |
| --- | --- |
| Product P/N: | APS25A4B (7)032G-8A (B)TM1G (W) |
| Firmware Version: | SFDG108A (B) |
| Controller: | SCJC72 |
| Qualification: | Qual all |
| Test Result: | PASS |

| APPROVAL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| QT | Date | Signature | Approved by | Date | Signature |
| Alex Lin | 2017/06/30 | Alex Lin | Danny Chen | 2017/06/30 | Danny Chen |

Table Of Content

SUMMARY RESULT	3

A.	Function Test With M/B : ASUS H97M-E	4

1.	Basic Function Test	4

2.	Format Test	4

3.	Burn In Test	4

4.	Reboot Test	4

5.	Install System Test	4

6.	Test Result	4

B.	Function Test With M/B : Avalue EMX QM77-A1R	5

1.	Basic Function Test	5

2.	Install System Test	5

3.	Test Result	5

C.	Performance Test With M/B : ASUS Z170-A	6

D.	Performance Test With M/B : Asus Maximus VI Hero	7

E.	Platform Compatibility Test	8

F.	Additional Qualification:	9

1.	Environmental	9

2.	Certification	9

SUMMARY RESULT

|  |  |  | PC1 |  |  |  | PC2 |  |  | PC3 |  |  |  | PC4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Product |  |  | SFD 25A-M 32GB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| System |  |  | Windows 10 |  |  |  | Fedora kernel 3.3.4 -5 fc17 |  |  |  |  |  |  |  |  |  |
| M/B | Name |  | H97M-E |  |  |  | QM77-A1R |  |  |  |  |  |  |  |  |  |
|  | Vendor |  | ASUS |  |  |  | Avalue EMX |  |  |  |  |  |  |  |  |  |
|  | Chipset |  | Intel H97 |  |  |  | Intel QM77 |  |  |  |  |  |  |  |  |  |
|  | Interface |  | SATA III |  |  |  | SATA III |  |  |  |  |  |  |  |  |  |
|  | BIOS |  | 2602 |  |  |  | V1.1.0 |  |  |  |  |  |  |  |  |  |
| Test Result |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Basic Function Test |  |  | V |  |  |  | V |  |  |  |  |  |  |  |  |  |
| Burn In Test | Comp32 |  | V |  |  |  | -- |  |  |  |  |  |  |  |  |  |
|  | PassMark Burn In Test Pro V8.0 |  | V |  |  |  | -- |  |  |  |  |  |  |  |  |  |
|  | PassMark Burn In Test Pro V3.0.1007 |  | -- |  |  |  | -- |  |  |  |  |  |  |  |  |  |
| Warm Boot Test |  |  | V |  |  |  | -- |  |  |  |  |  |  |  |  |  |
| Cold Boot Test |  |  | V |  |  |  | -- |  |  |  |  |  |  |  |  |  |
|  |  |  | -- |  |  |  | -- |  |  |  |  |  |  |  |  |  |
|  |  |  | -- |  |  |  | -- |  |  |  |  |  |  |  |  |  |
| Install System Test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Windows Emb. 7 Standard | Windows Emb. 8 Standard |  | Windows Emb. 8.1 Industry | Windows 10 | Fedora kernel 3.3.4 -5 fc17 |  | Ubuntu kernel 3.13.0 -32 | Red Hat kernel 2.6.32 -431.e16 |  |  |  |  |  |  |  |
| Result | V | V |  | V | V | V |  | V | V |  |  |  |  |  |  |  |
| Remark: Test Result — PASS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Function Test With M/B : ASUS H97M-E

| Testing Product | SFD 25A-M 32GB |  |  | Testing Engineer |  | Alex Lin |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Environment | M/B: | ASUS | Chipset: |  | Intel H97 |  | BIOS: | 2602 |
|  |  | H97M-E | Interface: |  | SATA III |  |  |  |

Basic Function Test

|  | Copy | Delete | Restart | Compare | Suspend (S3) | Hibernate (S4) | LED |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Windows 10 | V | V | V | V | V | V | V |

Format Test

|  | FAT32 | NTFS |
| --- | --- | --- |
| Windows 10 | 29.7GB | 29.8GB |

Burn In Test

|  | Comp32 15 hours | Burn In Test V8.0 Pro 168 hours |
| --- | --- | --- |
| Windows 10 | V | V |

Reboot Test

|  | Warm Boot 6000 Cycles | Cold Boot 6000 Cycles |
| --- | --- | --- |
| Windows 10 | V | V |

Install System Test

| Install System | Result |
| --- | --- |
| Install Windows Embedded Standard 7 System | V |
| Install Windows Embedded 8 Standard System | V |
| Install Windows Embedded 8.1 Industry Pro System | V |
| Windows 10 Multiple Edition | V |
| Windows 10 Enterprise | V |

Test Result

Pass

Function Test With M/B : Avalue EMX QM77-A1R

| Testing Product | SFD 25A-M 32GB |  |  | Testing Engineer |  | Alex Lin |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Environment | M/B: | Avalue | Chipset: |  | Intel QM77 |  | BIOS: | V1.1.0 |
|  |  | EMX QM77-A1R | Interface: |  | SATA III |  |  |  |

Basic Function Test

|  | Copy | Delete | Restart | Compare | Suspend (S3) | Hibernate (S4) | LED |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fedora kernel 3.3.4 -5 fc17 | V | V | V | V | V | V | V |

Install System Test

| Install System | Result |
| --- | --- |
| Install Windows Embedded Standard 7 System | V |
| Install Windows Embedded 8 Standard System | V |
| Install Windows Embedded 8.1 Industry Pro System | V |
| Install Windows 7 System | V |
| Install Windows 8.1 System | V |
| Install Fedora kernel 3.3.4 -5 System | V |
| Install Ubuntu kernel 3.13.0 -32 System | V |
| Install Red Hat Kernel 2.6.32 -431.e16 System | V |

Test Result

Pass

Performance Test With M/B : ASUS Z170-A

| Testing Product | SFD 25A-M 32GB |  |  | Testing Engineer |  | Alex Lin |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Environment | M/B: | ASUS | Chipset: |  | Intel Z170 |  | BIOS: | 1302 |
|  |  | Z170-A | Interface: |  | SATA III |  |  |  |
| Testing Software | CrystalMark 3.0.2 |  |  |  |  |  |  |  |

| Test Function : BurnIn Test 168hrs |  |
| --- | --- |
| Before | After |

Performance Test With M/B : Asus Maximus VI Hero

| Testing Product | SFD 25A-M 32GB |  |  | Testing Engineer |  | Alex Lin |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Environment | M/B: | ASUS | Chipset: |  | Intel Z87 |  | BIOS: | 0224 |
|  |  | Maximus VI Hero | Interface: |  | SATA III |  |  |  |
| Testing Software | Ubuntu 14.10 Benchmark |  |  |  |  |  |  |  |

Performance Before the Test

Platform Compatibility Test

| Platform | Chipset | M/B | Interface | System | Result |
| --- | --- | --- | --- | --- | --- |
| PC1 | Intel Z68 | Asus P8Z68-M Pro | SATA III | Win8.1 | V |
| PC2 | Intel Z77 | Gigabyte IVB | SATA III | Win8.1 | V |
| PC3 | Intel Z87 | Asus Maximus VI Hero | SATA III | Win8.1 | V |
| PC4 | Intel Z77 | GA-Z77X-D3H | SATA III | Win8.1 | V |
| PC5 | Intel QM77 | Avalue EMX QM77-A1R | SATA III | Win8.1 | V |
| PC6 | AMD 790FX | Asus M4A79T Deluxe | SATA III | Win8.1 | V |
| PC7 | AMD 790GX | Asus M4A78T-E | SATA III | Win8.1 | V |
| PC8 | AMD A88X | Asus A88XM-A | SATA III | Win8.1 | V |

Additional Qualification:

Environmental

- RoHS Standard  2011/65/EU , 2015/863/EU

- Other Environmental Standard  EC/1907/2006

- WEEE (By Product)  2012/19/EU

- Battery Standard  N/A

- Packing Standard  94/62/EC

- Others  N/A

Certification

- CE     No.

- FCC    No.

- BSMI   No.

- VCCC   No.

- C-tick   No.

- Other