# Product Qualification

# --- Compatibility Test ---

| Product Name: | SFD 25A 64GB HS |
| --- | --- |
| Product P/N: | 80.D1DU8.1T3B2BB (80.D1DU8.1T012BB) |
| Firmware Version: | SFDH103B (20140627) |
| Description: | SFD 25A 64GB HS |
| Controller: | Apacer SCJC72 |
| Flash Type: | TH58TEG6H2HBAMC |
| PCB Version: | 48.28262.0020B (E3) |
| Qualification: | Qual FW |
| Test Result: | Pass |

| APPROVAL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| QT | Date | Signature | Approved by | Date | Signature |
| Alex Lin | 2014/07/31 | Alex Lin | Danny Chen | 2014/07/31 | Danny Chen |

Table Of Content

SUMMARY RESULT	3

A.	Function Test With M/B : Gigabyte GA-Z77X-D3H	4

1.	Basic Function Test	4

2.	Format Test	4

3.	Burn In Test	4

4.	Reboot Test	4

5.	Install System Test	4

6.	Test Result	4

B.	Function Test With M/B : Asus M4A89GTD PRO USB3	5

1.	Basic Function Test	5

2.	Format Test	5

3.	Burn In Test	5

4.	Reboot Test	5

5.	Install System Test	5

6.	Test Result	5

C.	Function Test With M/B : Avalue EMX A55E	6

1.	Basic Function Test	6

2.	Format Test	6

3.	Burn In Test	6

4.	Reboot Test	6

5.	Power Failure Test	6

6.	Install System Test	6

7.	Test Result	6

D.	Performance Test With M/B : Asus P8Z77-V PRO	7

E.	Performance Test With M/B : Asus Maximus VI Hero	8

F.	Platform Compatibility Test	9

G.	Additional Qualification:	10

1.	Environmental	10

2.	Certification	10

SUMMARY RESULT

|  |  |  | PC1 |  |  | PC2 |  | PC3 |  |  | PC4 |  |  | PC5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Product |  |  | SFD 25A 64GB HS |  |  |  |  |  |  |  |  |  |  |  |  |
| System |  |  | Windows 7 Windows 8 |  |  | Windows 7 Windows 8 |  | Windows 7 Fedora 17 |  |  |  |  |  |  |  |
| M/B | Name |  | GA-Z77X-D3H |  |  | M4A89GTD PRO USB3 |  | ECM-A55E |  |  |  |  |  |  |  |
|  | Vendor |  | Gigabyte |  |  | Asus |  | Avalue |  |  |  |  |  |  |  |
|  | Chipset |  | Intel Z77 |  |  | AMD 890GX / SB850 |  | AMD A55E |  |  |  |  |  |  |  |
|  | Interface |  | SATA III |  |  | SATA III |  | SATS II |  |  |  |  |  |  |  |
|  | BIOS |  | F16 |  |  | 3207 |  | V1.26 |  |  |  |  |  |  |  |
| Test Result |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Basic Function Test |  |  | V |  |  | V |  | V |  |  |  |  |  |  |  |
| Burn In Test | Comp32 |  | V |  |  | V |  | V |  |  |  |  |  |  |  |
|  | Iometer 2006.07.27RC3 |  | V |  |  | V |  | V |  |  |  |  |  |  |  |
|  | PassMark Burn In Test Pro V6.0.1018 |  | V |  |  | V |  | V |  |  |  |  |  |  |  |
| Warm Boot Test |  |  | V |  |  | V |  | V |  |  |  |  |  |  |  |
| Cold Boot Test |  |  | -- |  |  | -- |  | V |  |  |  |  |  |  |  |
| Power Failure Test |  |  | -- |  |  | -- |  | V |  |  |  |  |  |  |  |
| Temp 60℃ Burn In Test |  |  | -- |  |  | -- |  | V |  |  |  |  |  |  |  |
| Install System Test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Windows CE | Windows XP-E |  | Windows Emb. 7 Standard | Windows Emb. 8.1 Industry |  | Windows XP |  | Windows 7 | Windows 8.1 |  | Fedora 17 | Ubuntu 12.04 |  | Red Hat 6.3 |
| Result | V | V |  | V | V |  | V |  | V | V |  | V | V |  | V |
| Remark: Test Result — PASS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Function Test With M/B : Gigabyte GA-Z77X-D3H

| Testing Product | SFD 25A 64GB HS |  |  | Testing Engineer |  | AlexLin |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Environment | M/B: | Gigabyte | Chipset: |  | Intel Z77 |  | BIOS: | F16 |
|  |  | GA-Z77X-D3H | Interface: |  | SATA III |  |  |  |

Basic Function Test

|  | Copy | Delete | Restart | Compare | Suspend (S3) | Hibernate (S4) | LED |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Win 7 | V | V | V | V | V | V | V |
| Win 8 | V | V | V | V | V | V | V |

Format Test

|  | FAT32 | NTFS | exFAT |
| --- | --- | --- | --- |
| Win 7 | 29.7GB | 29.8GB | -- |
| Win 8 | 29.7GB | 29.8GB | -- |

Burn In Test

|  | Comp32 15 hours | Iometer(2006.07.27-RC3) 15 hours | Burn In Test V6.0 Pro 48 hours |
| --- | --- | --- | --- |
| Win 7 | V | V | V |

Reboot Test

|  | Warm Boot 2000 Cycles |
| --- | --- |
| Win 7 | V |

Install System Test

| Install System | Result |
| --- | --- |
| Install Windows CE System | V |
| Install Windows XP-E System | V |
| Install Windows Embedded Standard 7 System | V |
| Install Windows Embedded 8 Standard System | V |
| Install Windows Embedded 8.1 Industry Pro System | V |
| Install Windows XP System | V |
| Install Windows 7 System | V |
| Install Windows 8.1 System | V |
| Install Fedora 17 System | V |
| Install Ubuntu 12.04 System | V |
| Install Red Hat 6.3 System | V |

Test Result

Pass

Function Test With M/B : Asus M4A89GTD PRO USB3

| Testing Product | SFD 25A 64GB HS |  |  | Testing Engineer |  | AlexLin |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Environment | M/B: | ASUS | Chipset: |  | AMD 890GX / SB850 |  | BIOS: | 3027 |
|  |  | M4A89GTD PRO USB3 | Interface: |  | SATA III |  |  |  |

Basic Function Test

|  | Copy | Delete | Restart | Compare | Suspend (S3) | Hibernate (S4) | LED |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Win XP | V | V | V | V | V | V | V |
| Win 7 | V | V | V | V | V | V | V |

Format Test

|  | FAT32 | NTFS | exFAT |
| --- | --- | --- | --- |
| Win XP | 29.7GB | 29.8GB | -- |
| Win 7 | 29.7GB | 29.8GB | -- |

Burn In Test

|  | Comp32 15 hours | Iometer(2006.07.27-RC3) 15 hours | Burn In Test V6.0 Pro 48 hours |
| --- | --- | --- | --- |
| Win XP | V | V | V |

Reboot Test

|  | Warm Boot 2000 Cycles |
| --- | --- |
| Win XP | V |

Install System Test

| Install System | Result |
| --- | --- |
| Install Windows CE System | V |
| Install Windows XP-E System | V |
| Install Windows Embedded Standard 7 System | V |
| Install Windows Embedded 8 Standard System | V |
| Install Windows Embedded 8.1 Industry Pro System | V |
| Install Windows XP System | V |
| Install Windows 7 System | V |
| Install Windows 8.1 System | V |
| Install Fedora 17 System | V |
| Install Ubuntu 12.04 System | V |
| Install Red Hat 6.3 System | V |

Test Result

Pass

Function Test With M/B : Avalue EMX A55E

| Testing Product | SFD 25A 64GB HS |  |  | Testing Engineer |  | Alex Lin |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Environment | M/B: | Avalue | Chipset: |  | AMD A55E |  | BIOS: | V1.26 |
|  |  | EMX A55E | Interface: |  | SATA III |  |  |  |

Basic Function Test

|  | Copy | Delete | Restart | Compare | Suspend (S3) | Hibernate (S4) | LED |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Win 7 | V | V | V | V | V | V | V |
| Fedora 17 | V | V | V | V | V | V | V |

Format Test

|  | FAT32 | NTFS | exFAT |
| --- | --- | --- | --- |
| Win 7 | 29.7GB | 29.8GB | -- |

Burn In Test

|  | Comp32 15 hours | Iometer(2006.07.27-RC3) 15 hours | Burn In Test V6.0 Pro 48 hours | Burn In Test V6.0 Pro Temp 60℃, 48 hours |
| --- | --- | --- | --- | --- |
| Win 7 | V | V | V | V |

Reboot Test

|  | Warm Boot 2000 Cycles | Cold Boot 2000 Cycle |
| --- | --- | --- |
| Win 8 | V | V |

Power Failure Test

|  | 2000 Cycles |
| --- | --- |
| Linux | V |

Install System Test

| Install System | Result |
| --- | --- |
| Install Windows CE System | V |
| Install Windows XP-E System | V |
| Install Windows Embedded Standard 7 System | V |
| Install Windows Embedded 8 Standard System | V |
| Install Windows Embedded 8.1 Industry Pro System | V |
| Install Windows XP System | V |
| Install Windows 7 System | V |
| Install Windows 8.1 System | V |
| Install Fedora 17 System | V |
| Install Ubuntu 12.04 System | V |
| Install Red Hat 6.3 System | V |

Test Result

Pass

Performance Test With M/B : Asus P8Z77-V PRO

| Testing Product | SFD 25A 64GB HS |  |  | Testing Engineer |  | Alex Lin |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Environment | M/B: | ASUS | Chipset: |  | Intel Z77 |  | BIOS: | 1206 |
|  |  | P8Z77-V PRO | Interface: |  | SATA III |  |  |  |
| Testing Software | CrystalMark 3.0.2 |  |  |  |  |  |  |  |

| Performance Before the Test | Performance After the Test |
| --- | --- |

Performance Test With M/B : Asus Maximus VI Hero

| Testing Product | SFD 25A 64GB HS |  |  | Testing Engineer |  | Alex Lin |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Environment | M/B: | ASUS | Chipset: |  | Intel Z87 |  | BIOS: | 0224 |
|  |  | Maximus VI Hero | Interface: |  | SATA III |  |  |  |
| Testing Software | Ubuntu 14.10 Benchmark |  |  |  |  |  |  |  |

Performance Before the Test

Platform Compatibility Test

| Platform Information |  |  |  | Test Item |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chipset | M/B | Interface | System | Format | Copy | Delete | Compare | Restart | Suspend |
| Intel G35 | Asus P5E-VM HDMI | SATA II | Win7 | V | V | V | V | V | V |
| Intel P55 | Asus P7P55D | SATA II | Win7 | V | V | V | V | V | V |
| Intel H67 | Gigabyte PH67A-D3-B3 | SATA II | Win8 | V | V | V | V | V | V |
| Intel Z68 | Asus P8Z68-M Pro | SATA II | Win8 | V | V | V | V | V | V |
| Intel Z77 | Gigabyte IVB | SATA II | Win7 | V | V | V | V | V | V |
|  |  | SATA III | Win7 | V | V | V | V | V | V |
| Intel Z87 | Asus Maximus VI Hero | SATA III | Win8.1 | V | V | V | V | V | V |
| Intel Z77 | GA-Z77X-D3H | SATA III | Win8 | V | V | V | V | V | V |
| Intel QM77 | Avalue EMX QM77-A1R | SATA III | Win8 | V | V | V | V | V | V |
| Intel Core i3-4010U | ntel NUC D34010WYK | SATA III | Win8 | V | V | V | V | V | V |
| AMD 780G | Ausu M3A78-EM | SATA II | Win7 | V | V | V | V | V | V |
| AMD 785G | Asus M4A785D-M Pro | SATA II | Win7 | V | V | V | V | V | V |
| AMD 790FX | Asus M4A79T Deluxe | SATA II | Win7 | V | V | V | V | V | V |
| AMD 790GX | Asus M4A78T-E | SATA II | Win8.1 | V | V | V | V | V | V |
| AMD A55E | Avalue EMX-A55E | SATA II | Win7 | V | V | V | V | V | V |
| AMD A88X | Asus A88XM-A | SATA III | Win8.1 | V | V | V | V | V | V |

Additional Qualification:

Environmental

- RoHS Standard

- Other Environmental Standard

- WEEE (By Product)

- Battery Standard

- Packing Standard

- Others

Certification

- CE     No.

- FCC    No.

- BSMI   No.

- VCCC   No.

- C-tick   No.

- Other