[image: image3.jpg]Apacer

Access the best




PAGE  


Product Qualification

--- Compatibility Test ---

	Product Name:
	M.2 T42 A120-M 128GB

	Product P/N:
	81.5AGT2.2T000BA

	Firmware Version:
	SFDK001A (093.038)

	Description:
	M.2 T42 A120-M 128GB

	Controller:
	Jmicron JM609

	Flash Type:
	TH58TEG9DDKBA8H

	PCB Version:
	48.26260.0100B(E3)

	Qualification:
	Qual Flash

	Test Result:
	Pass


	APPROVAL

	QT
	Date
	Signature
	Approved by
	Date
	Signature

	Alex Lin
	 2014/12/25
	Alex Lin
	Danny Chen
	2014/12/25
	Danny Chen


Table Of Content

3SUMMARY RESULT


4A.
Function Test With M/B : ASUS P8Z68-M PRO


41.
Basic Function Test


42.
Format Test


43.
Burn In Test


44.
Reboot Test


45.
Install System Test


46.
Test Result


5B.
Performance Test With M/B : Asus P8Z77-V PRO


6C.
Additional Qualification:


61.
Environmental


62.
Certification




SUMMARY RESULT
	
	PC1
	PC2
	PC3
	PC4
	PC5

	Testing
Product
	M.2 T42 A120-M 128GB
	
	

	System
	Windows 7
Windows 8
	
	
	
	

	M/B
	Name
	P8Z68-M PRO
	
	
	
	

	
	Vendor
	ASUS
	
	
	
	

	
	Chipset
	Intel Z68
	
	
	
	

	
	Interface
	SATA III
	
	
	
	

	
	BIOS
	0604
	
	
	
	

	Test Result

	Basic Function Test
	V
	
	
	
	

	Burn

In

Test
	Comp32
	V
	
	
	
	

	
	Iometer

2006.07.27RC3
	V
	
	
	
	

	
	PassMark

Burn In Test

Pro V6.0.1018
	V
	
	
	
	

	Warm Boot Test
	V
	
	
	
	

	Cold Boot Test
	--
	
	
	
	

	Power Failure Test
	--
	
	
	
	

	Temp 60℃
Burn In Test
	--
	
	
	
	

	Install System Test

	
	Windows

CE
	Windows

XP-E
	Windows

Emb. 7
Standard
	Windows

Emb. 8.1
Industry
	Windows

XP
	Windows

7
	Windows

8.1
	Fedora

17
	Ubuntu

12.04
	Red Hat

6.3

	Result
	V
	V
	V
	V
	V
	V
	V
	V
	V
	V

	Remark:
Test Result — PASS



A. Function Test With M/B : ASUS P8Z68-M PRO
	Testing Product
	M.2 T42 A120-M 128GB
	Testing Engineer
	Alex Lin

	Testing Environment
	M/B:
	ASUS
	Chipset:
	Intel Z68
	BIOS:
	0604

	
	
	P8Z68-M PRO
	Interface:
	SATA II


1. Basic Function Test

	
	Copy
	Delete
	Restart
	Compare
	Suspend
(S3)
	Hibernate
(S4)
	LED

	Win 7
	V
	V
	V
	V
	V
	V
	V

	Win 8
	V
	V
	V
	V
	V
	V
	V


2. Format Test

	
	FAT32
	NTFS
	exFAT

	Win 7
	--
	119GB
	--

	Win 8
	--
	119GB
	--


3. Burn In Test

	
	Comp32

15 hours
	Iometer(2006.07.27-RC3)
15 hours
	Burn In Test V6.0 Pro
48 hours

	Win 7
	V
	V
	V


4. Reboot Test

	
	Warm Boot 2000 Cycles

	Win 8
	V


5. Install System Test

	Install System
	Result

	Install Windows CE System
	V

	Install Windows XP-E System
	V

	Install Windows Embedded Standard 7 System
	V

	Install Windows Embedded 8 Standard System
	V

	Install Windows Embedded 8.1 Industry Pro System
	V

	Install Windows XP System
	V

	Install Windows 7 System
	V

	Install Windows 8.1 System
	V

	Install Fedora 17 System
	V

	Install Ubuntu 12.04 System
	V

	Install Red Hat 6.3 System
	V


6. Test Result
	Pass




B. Performance Test With M/B : Asus P8Z77-V PRO
	Testing Product
	M.2 T42 A120-M 128GB
	Testing Engineer
	Alex Lin

	Testing Environment
	M/B:
	ASUS
	Chipset:
	Intel Z77
	BIOS:
	1206

	
	
	P8Z77-V PRO
	Interface:
	SATA III

	Testing Software
	CrystalMark 3.0.2



	Performance Before the Test
	Performance After the Test

	[image: image1.jpg]E CrystalDiskMark 302 x64 (o]

Ele Edit Theme Help Langusge

5[v] [1000M8 [V] [E: 0% (0/119GB)
Read [MB/s]  Write [MB/s]

= 434.4 141.5
524 /32618 1103.0
w122.79 45l67
W 145(59 72/97

All





	[image: image2.jpg]= CrystalDiskMark 302

All

Seq

512K

4K

4K
Qp32

Ele Edt Theme Help Langusge

5[] [1000MB [V] [E: 0% (0/119GB)
Read [MB/s]  Write [MB/s]

442.9 1132.1
332.7 1101.3
22.79 14130
4542 62143







C.  Additional Qualification:

1. Environmental

1. RoHS Standard                                                  
2. Other Environmental Standard                                      

3. WEEE (By Product)                                              
4. Battery Standard                                                 
5. Packing Standard                                                 
6. Others                                                         
2. Certification

1. ( CE     No.                                                
2. ( FCC    No.                                                
3. ( BSMI   No.                                                
4. ( VCCC   No.                                               
5. ( C-tick   No.                                                
6. ( Other                                                      
1
 Form No: PCRD002-F05-02
PAGE  
2
 Form No: PCRD002-F05-02

