[image: image4.jpg]Apacer

Access the best




PAGE  


Product Qualification

--- Compatibility Test ---

	Product Name:
	SDM5A-M 22P 180D 16GB

	Product P/N:
	81.F1CF4.2T006BA

	Firmware Version:
	SFDK001A (20141128)

	Description:
	SDM5A-M 22P 180D 16GB

	Controller:
	Jmicron JM609

	Flash Type:
	TC58TEG5DCKTA000

	PCB Version:
	48.26257.0160B

	Qualification:
	Qual Flash

	Test Result:
	Pass


	APPROVAL

	QT
	Date
	Signature
	Approved by
	Date
	Signature

	Alex Lin
	 2015/02/12
	Alex Lin
	Danny Chen
	2015/02/12
	Danny Chen


Table Of Content

3SUMMARY RESULT


4A.
Function Test With M/B : Gigabyte GA-Z77X-D3H


41.
Basic Function Test


42.
Format Test


43.
Burn In Test


44.
Reboot Test


45.
Install System Test


46.
Test Result


5B.
Function Test With M/B : Asus M4A89GTD PRO USB3


51.
Basic Function Test


52.
Format Test


53.
Burn In Test


54.
Reboot Test


55.
Install System Test


56.
Test Result


6C.
Function Test With M/B : Avalue EMX QM77-A1R


61.
Basic Function Test


62.
Format Test


63.
Burn In Test


64.
Reboot Test


65.
Power Failure Test


66.
Install System Test


67.
Test Result


7D.
Performance Test With M/B : Asus P8Z77-V PRO


8E.
Performance Test With M/B : Asus Maximus VI Hero


9F.
Platform Compatibility Test


10G.
Additional Qualification:


101.
Environmental


102.
Certification




SUMMARY RESULT
	
	PC1
	PC2
	PC3
	PC4
	PC5

	Testing
Product
	SDM5A-M 22P 180D 16GB
	
	

	System
	Windows 7
Windows 8
	Windows 7

Windows 8
	Windows 7

Fedora 17
	
	

	M/B
	Name
	GA-Z77X-D3H
	M4A89GTD PRO USB3
	EMX QM77-A1R
	
	

	
	Vendor
	Gigabyte
	Asus
	Avalue
	
	

	
	Chipset
	Intel Z77
	AMD 890GX / SB850
	Intel QM77
	
	

	
	Interface
	SATA III
	SATA III
	SATS III
	
	

	
	BIOS
	F16
	3207
	V1.10
	
	

	Test Result

	Basic Function Test
	V
	V
	V
	
	

	Burn

In

Test
	Comp32
	V
	V
	V
	
	

	
	Iometer

2006.07.27RC3
	V
	V
	V
	
	

	
	PassMark

Burn In Test

Pro V6.0.1018
	V
	V
	V
	
	

	Warm Boot Test
	V
	V
	V
	
	

	Cold Boot Test
	--
	--
	V
	
	

	Power Failure Test
	--
	--
	V
	
	

	Temp 60℃
Burn In Test
	--
	--
	V
	
	

	Install System Test

	
	Windows

CE
	Windows

XP-E
	Windows

Emb. 7
Standard
	Windows

Emb. 8.1
Industry
	Windows

XP
	Windows

7
	Windows

8.1
	Fedora

17
	Ubuntu

12.04
	Red Hat

6.3

	Result
	V
	V
	V
	V
	V
	V
	V
	V
	V
	V

	Remark:
Test Result — PASS



A. Function Test With M/B : Gigabyte GA-Z77X-D3H
	Testing Product
	SDM5A-M 22P 180D 16GB
	Testing Engineer
	Alex Lin

	Testing Environment
	M/B:
	Gigabyte
	Chipset:
	Intel Z77
	BIOS:
	F16

	
	
	GA-Z77X-D3H
	Interface:
	SATA III


1. Basic Function Test

	
	Copy
	Delete
	Restart
	Compare
	Suspend
(S3)
	Hibernate
(S4)
	LED

	Win 7
	V
	V
	V
	V
	V
	V
	V

	Win 8
	V
	V
	V
	V
	V
	V
	V


2. Format Test

	
	FAT32
	NTFS
	exFAT

	Win 7
	14.8GB
	14.9GB
	--

	Win 8
	14.8GB
	14.9GB
	--


3. Burn In Test

	
	Comp32

15 hours
	Iometer(2006.07.27-RC3)
15 hours
	Burn In Test V6.0 Pro
48 hours

	Win 7
	V
	V
	V


4. Reboot Test

	
	Warm Boot 2000 Cycles

	Win 7
	V


5. Install System Test

	Install System
	Result

	Install Windows CE System
	V

	Install Windows XP-E System
	V

	Install Windows Embedded Standard 7 System
	V

	Install Windows Embedded 8 Standard System
	V

	Install Windows Embedded 8.1 Industry Pro System
	V

	Install Windows XP System
	V

	Install Windows 7 System
	V

	Install Windows 8.1 System
	V

	Install Fedora 17 System
	V

	Install Ubuntu 12.04 System
	V

	Install Red Hat 6.3 System
	V


6. Test Result
	Pass




B. Function Test With M/B : Asus M4A89GTD PRO USB3
	Testing Product
	SDM5A-M 22P 180D 16GB
	Testing Engineer
	Alex Lin

	Testing Environment
	M/B:
	ASUS
	Chipset:
	AMD 890GX / SB850
	BIOS:
	3027

	
	
	M4A89GTD PRO USB3
	Interface:
	SATA III


1. Basic Function Test

	
	Copy
	Delete
	Restart
	Compare
	Suspend
(S3)
	Hibernate
(S4)
	LED

	Win XP
	V
	V
	V
	V
	V
	V
	V

	Win 7
	V
	V
	V
	V
	V
	V
	V


2. Format Test

	
	FAT32
	NTFS
	exFAT

	Win XP
	14.8GB
	14.9GB
	--

	Win 7
	14.8GB
	14.9GB
	--


3. Burn In Test

	
	Comp32

15 hours
	Iometer(2006.07.27-RC3)
15 hours
	Burn In Test V6.0 Pro
48 hours

	Win XP
	V
	V
	V


4. Reboot Test

	
	Warm Boot 2000 Cycles

	Win XP
	V


5. Install System Test

	Install System
	Result

	Install Windows CE System
	V

	Install Windows XP-E System
	V

	Install Windows Embedded Standard 7 System
	V

	Install Windows Embedded 8 Standard System
	V

	Install Windows Embedded 8.1 Industry Pro System
	V

	Install Windows XP System
	V

	Install Windows 7 System
	V

	Install Windows 8.1 System
	V

	Install Fedora 17 System
	V

	Install Ubuntu 12.04 System
	V

	Install Red Hat 6.3 System
	V


6. Test Result
	Pass




C. Function Test With M/B : Avalue EMX QM77-A1R
	Testing Product
	SDM5A-M 22P 180D 16GB
	Testing Engineer
	Alex Lin

	Testing Environment
	M/B:
	Avalue
	Chipset:
	Intel QM77
	BIOS:
	V1.10

	
	
	EMX QM77-A1R
	Interface:
	SATA III


1. Basic Function Test

	
	Copy
	Delete
	Restart
	Compare
	Suspend
(S3)
	Hibernate
(S4)
	LED

	Win 7
	V
	V
	V
	V
	V
	V
	V

	Fedora 17
	V
	V
	V
	V
	V
	V
	V


2. Format Test

	
	FAT32
	NTFS
	exFAT

	Win 7
	14.8GB
	14.9GB
	--


3. Burn In Test

	
	Comp32

15 hours
	Iometer(2006.07.27-RC3)
15 hours
	Burn In Test V6.0 Pro
48 hours
	Burn In Test V6.0 Pro
Temp 60℃, 48 hours

	Win 7
	V
	V
	V
	V


4. Reboot Test

	
	Warm Boot 2000 Cycles
	Cold Boot 2000 Cycle

	Win 8
	V
	V


5.  Power Failure Test

	
	2000 Cycles

	Linux
	V


6. Install System Test

	Install System
	Result

	Install Windows CE System
	V

	Install Windows XP-E System
	V

	Install Windows Embedded Standard 7 System
	V

	Install Windows Embedded 8 Standard System
	V

	Install Windows Embedded 8.1 Industry Pro System
	V

	Install Windows XP System
	V

	Install Windows 7 System
	V

	Install Windows 8.1 System
	V

	Install Fedora 17 System
	V

	Install Ubuntu 12.04 System
	V

	Install Red Hat 6.3 System
	V


7. Test Result
	Pass




D. Performance Test With M/B : Asus P8Z77-V PRO
	Testing Product
	SDM5A-M 22P 180D 16GB
	Testing Engineer
	Alex Lin

	Testing Environment
	M/B:
	ASUS
	Chipset:
	Intel Z77
	BIOS:
	1206

	
	
	P8Z77-V PRO
	Interface:
	SATA III

	Testing Software
	CrystalMark 3.0.2



	Performance Before the Test
	Performance After the Test

	[image: image1.jpg]= CrystalDiskMark 302 x64 (=]

Ele Edit Theme Help Langusge

5 [+] [1000M8 [~] [E: 1% (0/15GB) =]
Read [MB/s]  Write [MB/s]

| 2342 |42i10
4 1201.7 ¥37.83
s21.52 }15.91
140.83 18575

All





	[image: image2.jpg]|5 CrystalDiskMark 3.0.2

Ele Edit Theme Help Langusge

All

Seq

512K

4K

4K
Qp32

5 [+] [1000MB [~] [E: 1% (0/15GB) =]

Read [MB/s]  Write [MB/s]

238.0 43123
207.2 139.61
24.57 32,19
4131 34,08







E. Performance Test With M/B : Asus Maximus VI Hero
	Testing Product
	SDM5A-M 22P 180D 16GB
	Testing Engineer
	Alex Lin

	Testing Environment
	M/B:
	ASUS
	Chipset:
	Intel Z87
	BIOS:
	0224

	
	
	Maximus VI Hero
	Interface:
	SATA III

	Testing Software
	Ubuntu 14.10

Benchmark


	Performance Before the Test

	[image: image3.png]Benchmark

o ms e
ey 03ms
376wt 03
329wt 07ms
252w osms
35w osms
198wt o4
161wt o03ms

Disk or Device Partition 1 of 16 GB Disk — 16GB SATA Flash Drive [SFDKOO1A] (/dey

Last Benchmarked Thu 12 Feb 2015 O1.

7 PMEST (Less than a minute ago)
Sample Size 10.0 MiB (10,485,760 bytes)

Average Read Rate 450.1 MB/s (100 samples)

Average Write Rate 43.0 MB/s (100 samples)

Average Access Time 0.06 msec (1000 samples)

Start Benchmark... Close







F. Platform Compatibility Test
	Platform Information
	Test Item

	Chipset
	M/B
	Interface
	System
	Format
	Copy
	Delete
	Compare
	Restart
	Suspend

	Intel G35
	Asus P5E-VM HDMI
	SATA II
	Win7
	V
	V
	V
	V
	V
	V

	Intel P55
	Asus P7P55D
	SATA II
	Win7
	V
	V
	V
	V
	V
	V

	Intel H67
	Gigabyte PH67A-D3-B3
	SATA II
	Win8
	V
	V
	V
	V
	V
	V

	Intel Z68
	Asus P8Z68-M Pro
	SATA II
	Win8
	V
	V
	V
	V
	V
	V

	Intel Z77
	Gigabyte IVB
	SATA II
	Win7
	V
	V
	V
	V
	V
	V

	
	
	SATA III
	Win7
	V
	V
	V
	V
	V
	V

	Intel Z87
	Asus Maximus VI Hero
	SATA III
	Win8.1
	V
	V
	V
	V
	V
	V

	Intel Z77
	GA-Z77X-D3H
	SATA III
	Win8
	V
	V
	V
	V
	V
	V

	Intel QM77
	Avalue EMX QM77-A1R
	SATA III
	Win8
	V
	V
	V
	V
	V
	V

	AMD 780G
	Ausu M3A78-EM
	SATA II
	Win7
	V
	V
	V
	V
	V
	V

	AMD 785G
	Asus M4A785D-M Pro
	SATA II
	Win7
	V
	V
	V
	V
	V
	V

	AMD 790FX
	Asus M4A79T Deluxe
	SATA II
	Win7
	V
	V
	V
	V
	V
	V

	AMD 790GX
	Asus M4A78T-E
	SATA II
	Win8.1
	V
	V
	V
	V
	V
	V

	AMD A88X
	Asus A88XM-A
	SATA III
	Win8.1
	V
	V
	V
	V
	V
	V


G. Additional Qualification:

1. Environmental

1. RoHS Standard                                                  
2. Other Environmental Standard                                      

3. WEEE (By Product)                                              
4. Battery Standard                                                 
5. Packing Standard                                                 
6. Others                                                         
2. Certification

1. ( CE     No.                                                
2. ( FCC    No.                                                
3. ( BSMI   No.                                                
4. ( VCCC   No.                                               
5. ( C-tick   No.                                                
6. ( Other                                                      
1
 Form No: PCRD002-F05-02
PAGE  
8
 Form No: PCRD002-F05-02

